1. traffic_main.py (The Neural Core)

Use: This is the "brain" of the system. It performs real-time computer vision to detect how many vehicles are in each lane.

What it does:

Detection: Scans camera/video frames to count vehicles.

Logic: Calculates how long a green light should stay on (min(60s, count * 2.5s)).

Communication: Sends the signal states (e.g., G,R,R,Y) to your Arduino.

How to use:

Install dependencies: python3 -m pip install opencv-python pyserial numpy.

Connect your Arduino via USB (optional).

Run: python3 traffic_main.py.

2. yudhisthra_arduino.ino (The Hardware Interface)

Use: This is the firmware that runs on the physical Arduino board. It listens for instructions from the Python script and switches the actual LEDs on your traffic light model.

What it does:

Receives Data: Listens for the G,R,Y strings from the Python script.

Drives LEDs: Physically turns the LED pins HIGH or LOW.

Diagnostics: Runs a startup sequence to verify all lights are working.

How to use:

Open

yudhisthra_arduino.ino in the Arduino IDE.

Select your board (e.g., Arduino Uno) and the correct Port.

Click the Upload arrow to flash the code onto the board.

How They Work Together

Arduino waits for a signal.

Python sees traffic building up in Lane 1.

Python calculates 30 seconds of Green time and sends "G,R,R,R" over USB.

Arduino receives the command and switches the Lane 1 LED to Green.

TIP

If you don't have an Arduino connected, traffic_main.py will automatically switch to Software Simulation Mode, allowing you to test the logic using only your screen!